LAB TASKS (Lab # 02)

Note: Prepare only truth tables for the following statements. Along with this

document, also attach circuit images of the 3 tasks completed in class

(hardware lab).

Lab Task 1: Implement a NOT gate using NAND gate only.

Circuit Description:

 Use one NAND gate

 Connect both inputs together (A tied with A)

Boolean Expression: Y = (A · A)’ = A’

| A | A | (A.A)' |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 1 | 0 |

Lab Task 2: Implement an AND gate using NAND gates only (NAND–NAND

implementation).

Circuit Description:

 First NAND gives (A·B)’

 Second NAND acts as NOT (inputs tied)

Expression: Y = ((A·B)’)’ = A·B

| A | B | (A.B)' | ((A.B)')' | A.B |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |

Lab Task 3: Implement an OR gate using NAND gates only.

Circuit Description:

 Use two NAND gates as NOT on A and B

 Feed outputs to a NAND gate

Expression: Y = (A’·B’)’ = A + B

| A | B | A' | B' | A'.B' | (A'.B')' | A+B |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 |

Lab Task 4: Implement a NOT gate using NOR gate only.

Circuit Description: Tie both inputs of NOR gate

Expression: Y = (A + A)’ = A’

| A | A+A | (A+A)' | A' |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |

Lab Task 5: Implement an OR gate using NOR gates only (NOR–NOR).

Circuit Description:

 First NOR gives (A+B)’

 Second NOR acts as NOT

Expression: Y = ((A+B)’)’ = A + B

| A | B | A+B | (A+B)' | ((A+B)')' |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 |

Lab Task 6: Implement an AND gate using NOR gates only.

Circuit Description:

 Generate A’ and B’ using NOR

 NOR them again

Expression: Y = (A’ + B’)’ = A·B

| A | B | A.B | A' | B' | A'+B' | (A'+B')' |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 |

Lab Task 7: Implement the Boolean function using NAND only:

Y=(A+B)(B+C)

| A | B | C | A+B | B+C | (A+B)(B+C) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 |

Lab Task 8: Implement the Boolean function using NOR only:

Y=(AB+C)’

| A | B | C | AB | AB+C | (AB+C)' |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 |

Dear sir 3rd and 4th pics would be looking like same although both are same I have done my these two questions at once using more wire so both Boolean equations are applied on the one pic so both pic are same for your clarity I have provided both pics